Pyrocam III ActiveX User’s Manual

Table of Contents

1	Introduction	2

2	Using ActiveX	2

2.1	Microsoft Excel	2

2.2	Visual Basic (Visual Studio)	3

2.3	LabVIEW	4

3	Properties, Methods, and Events	4

3.1	Properties	4

3.1.1	isLabVIEW	4

3.1.2	NewFrame, HoldNewFrame	4

3.1.3	Data	5

3.2	Methods	5

3.2.1	Open	5

3.2.2	LoadSetup	5

3.2.3	Start	5

3.2.4	Stop	5

3.2.5	Ultracal	5

3.3	Events	6

3.3.1	OnNewFrame	6

# Introduction

The Pyrocam III ActiveX control provides simple control of Pyrocam III along with access to frame data, beam display image, results, statistics, and pass/fail indicators.

Using the Pyrocam III ActiveX control you can:

- Start and Stop collecting and processing new data frames

- Initiate an Ultracal cycle

- Read new data frames

Using the Pyrocam III ActiveX control you cannot:

- Detect or handle Pyrocam III errors or error messages

- Read or set the Pyrocam III configuration

# Using ActiveX

Many modern development languages such as Visual Basic, Visual C++, and Borland C++ Builder support ActiveX.  Many applications such as Microsoft Word, Microsoft Excel, and National Instruments LabVIEW also support ActiveX.  Development languages and applications all use different methods for accessing ActiveX controls.

Below we will briefly describe how to use the Pyrocam III ActiveX control in Microsoft Excel, Visual Basic, and LabVIEW.

## Microsoft Excel

Visual Basic for Applications (VBA) is integrated into Microsoft Excel.  You must create a VBA form in order to use the Pyrocam III ActiveX control in Microsoft Excel.  Follow these steps to use the Pyrocam III ActiveX control in Microsoft Excel:

- Create a new Workbook in Excel

- Go to the Visual Basic Editor.  On the Tools menu, select the Macro menu, then select Visual Basic Editor.  A new VBA window will open.

- Create a new form.  On the Insert menu, select UserForm.  A form is displayed in a new window.

- Add the Pyrocam III ActiveX control to the Controls Toolbox.  On the Tools menu, select Additional Controls…  Scroll down until you see PyrocamIIIActiveX Control.  Select the checkbox to the left of PyrocamIIIActiveX Control.  A new crosshatched icon appears on the Controls Toolbox.

- Add the Pyrocam III ActiveX control to the form.  Select the PyrocamIIIActiveX control on the Controls Toolbox.  Move the mouse pointer over the form and click the left mouse button.  A new control with the Pyrocam III icon appears on the form.  This new control has the default name PyrocamIIIActiveX1.

- Add other controls to the form such as Start/Stop/Ultracal buttons, etc.

- Initialize communication with Pyrocam III.  On the View menu, select Code.  A new window appears with an outline of the UserForm_Click() subroutine.  There are two list boxes at the top of the new window.  In the right list box, select Initialize.  A new subroutine called UserForm_Initialize() is created.  In this subroutine type “PyrocamIIIActiveX1.Open” without the quotation marks.  This statement initiates communication between the Pyrocam III ActiveX control and Pyrocam III.

- Respond to Pyrocam III ActiveX events.  In the left list box, select PyrocamIIIActiveX1.  A new subroutine called PyrocamIIIActiveX1_OnNewFrame() is created.  This subroutine is called every time the Pyrocam III collects a new frame of data.

- Do something during the event.  In PyrocamIIIActiveX1_OnNewFrame() subroutine type “Range(“B2”).Value = PyrocamIIIActiveX1.Results(0)” without the outer quotation marks.  This statement puts the frame Total result in cell B2.

An example Workbook,  PyrocamIIIActiveXExample.xls, can be found in “ActiveX\Examples\Excel” directory under the Pyrocam III installation directory.

## Visual Basic (Visual Studio)

In this section Visual Basic refers to the Visual Basic part of the Microsoft Visual Studio family of development products. Follow these steps to use the Pyrocam III ActiveX control in Microsoft Visual Basic:

- Add the Pyrocam III ActiveX control to the Project.  On the Project menu, select Components…  Scroll down until you see PyrocamIIIActiveX Control.  Select the checkbox to the left of PyrocamIIIActiveX Control.  A new Pyrocam III icon appears on the General Toolbox.

- Add the Pyrocam III ActiveX control to the form.  Select the PyrocamIIIActiveX control on the Controls Toolbox.  Move the mouse pointer over the form, hold the left mouse button, drag the mouse cursor down and right, finally release the mouse button.  A new control with the Pyrocam III icon appears on the form.  This new control has the default name PyrocamIIIActiveX1.

- Initialize communication with Pyrocam III.  Double click the form.  A new window appears with the subroutine Form_Load().  In this subroutine type “PyrocamIIIActiveX1.Open” without the quotation marks.  This statement initiates communication between the Pyrocam III ActiveX control and Pyrocam III.

- Respond to Pyrocam III ActiveX events. .  In the left list box, select PyrocamIIIActiveX1.  A new subroutine called PyrocamIIIActiveX1_OnNewFrame() is created.  This subroutine is called every time the Pyrocam III collects a new frame of data.

An example Visual Basic project, PyrocamIIIActiveXExample.vbp, can be found in “ActiveX\Examples\Visual Basic” directory under the Pyrocam III installation directory.

## LabVIEW

Follow these steps to use the Pyrocam III ActiveX control in National Instruments LabVIEW:

- Place an ActiveX reference on the Front Panel.  On the Controls palette, select ActiveX, and then select ActiveX Container (do not use Automation Refnum).  Place the control on the Front Panel.

- Reference the Pyrocam III ActiveX control.  Right click on the ActiveX Container and select Insert ActiveX Object…  Scroll down and select PyrocamIIIActiveX Control.  Click OK.

- Display the Diagram window.  On the Window menu, select Show Diagram.

- To access a Pyrocam III ActiveX property, right click on the PyrocamIIIActiveX node, select Create, select Property, and then select the desired property from the list.  Connect a wire from the PyrocamIIIActiveX Refnum to reference terminal of the property node.

- To call a Pyrocam III ActiveX method, right click on the PyrocamIIIActiveX node, select Create, select Method, and then select the desired property from the list.  Connect a wire from the PyrocamIIIActiveX Refnum to reference terminal of the method node.

- See the Events section of this document, and the OnNew examples for information on how to use Pyrocam III ActiveX events in LabVIEW.

Example VI’s are packaged in PyrocamIIIActiveX.llb, which can be found in the “ActiveX\Examples\LabVIEW” directory under the Pyrocam III installation directory.

# Properties, Methods, and Events

ActiveX components operate on the PME system, where PME is:

Properties  - Think of these as data items

Methods    - Functions you can call to perform some operation

Events	      - Notification when things happen

## Properties

### isLabVIEW

If you are using events in LabVIEW you must set isLabVIEW to true.  If you do not, the events will not fire in LabVIEW.

### NewFrame, HoldNewFrame

These properties can be used for polling when a new frame is collected by Pyrocam III and to hold the data, bitmap, picture, results, statistics, and pass/fail flags.

The NewFrame property is set TRUE each time the Pyrocam III collects a new frame of data.  You must reset NewFrame to FALSE to detect when the next new frame of data is collected.

If HoldNewFrame is TRUE, then the data, bitmap, picture, results, statistics, and pass/fail flags will not change until you reset NewFrame to FALSE.  (The Pyrocam III will continue to collecting and processing frames of data)

### Data

This property provides raw Pyrocam III data frames.

Data is a two dimensional array of doubles packaged as a Variant.  The leftmost dimension is the number of rows and has the range 0 to 123.  The rightmost dimension is the number of columns and has the range 0 to 123.  The Data is in the same order as displayed in the LBA-PC Beam Display Window, top to bottom, left to right.

## Methods

### Open

This method initiates communication between the Pyrocam III ActiveX control and Pyrocam III.  You must call this method before calling any other method or accessing any of the properties.  If you do not call this method then all the properties will be zero, methods will have no effect, and no events will fire.

### LoadSetup

This method causes the Pyrocam III Control Console to load the specified setup on the currently selected camera.  If the camera is collecting frames then it will be stopped, the new setup loaded, then data collection will be restarted.

### Start

This method starts the OnNewFrame event notification.

If Ultracal is not complete the Pyrocam III cannot start.  If there is any other problem, the Pyrocam III will not start and will display an error message on the Pyrocam III display.

### Stop

This method stops the OnNewFrame event notification.  The camera will continue to collect data but you will no longer receive the OnNewFrame event.

This method immediately cancels the Ultracal operation.  Any previous Ultracal is retained.

### Ultracal

This method is identical to clicking Ultracal! on the Pyrocam III. This method has no effect if a previous Ultracal operation is not complete.

If the Pyrocam III is collecting frames of data the Ultracal cannot start.  If there is any other problem, the Ultracal will not start and the Pyrocam III will display an error message on the Pyrocam III display.

This method returns the following:

-1		Pyrocam III not available

0		Ultracal started

1		Pyrocam III is collecting frames of data

The Ultracal operation runs for unknown amount of time depending on the camera and the Pyrocam III configuration.

## Events

All the events can fire each time the Pyrocam III collects a new frame of data.  If you are still processing an event when another frame is collected then the new event is not fired.  This is especially true when using LabVIEW.

NOTE:  If you are using events in LabVIEW you must set isLabVIEW to true.  If you do not, the events will not fire in LabVIEW.

OnNewData is specifically designed for LabVIEW.  This event is fired each time the Pyrocam III collects a new frame of data.

### OnNewFrame

This is a generic event fired each time the Pyrocam III collects a new frame of data.  No parameters are sent with this event.  Rather you read any desired property values while processing this event.  During this event you are guaranteed all properties correspond to the same frame of data.  After return from this event property values will change.

This event is useful in LabVIEW only for counting the number of frames collected by Pyrocam III.

Use the OnNewData event in LabVIEW if you need properties that all correspond to the same frame of data.

In Visual Basic you can read any desired properties during this event.  All properties correspond to the same frame of data.  After return from this event property values will change.